- Advanced NDT

3.1 Positive Material Identification (PMI)

AKMEC’s highly experienced PMI inspectors can provide on-the-spot results followed by the certification of results in a written report.  We also interpret and advise on the results. Specifically, PMI is used to confirm that the chemical composition of the metallic parts has the correct percentage of key elements, this ensures that material properties such as corrosion resistance meets the requirements.

The PMI is utilized for quality control and safety compliance and is an integral part of both production and asset integrity management across many industries including oil and gas, power generation, chemical, pharmaceutical, nuclear, aerospace, and metal fabrication.

ADVANTAGES

- A rapid non-destructive method utilized to verify that supplied materials conform to the proper standards and specifications

- Find potentially mixed-up alloys

- Identify if the wrong material has been used

- Ensure welded components have used the correct filler material

- Ensure that products and components have been manufactured using the correct alloy

3.2 Eddy Current Testing (ECT)

Eddy Current Testing (ECT) is a rapid and accurate technique used to detect discontinuities in the tubing, heat exchangers, condensers, steam generators, air coolers, feed water heaters wires, plates, etc. AKMEC uses electromagnetic induction to detect flaws in conductive materials. Eddy current testing can detect very small cracks in or near the surface of the material. The surfaces need minimal preparation, and physically complex geometries can be investigated using this method. Eddy current testing is also performed for alloy separation, for the determination of treatment conditions, for making electrical conductivity and for measuring coating thickness. The location of repair welds, girth welds, and seam welds may also be detected on ground-machined surfaces.

AKMEC has an in-house team of Eddy Current Testing professionals with deep expertise in inspecting a number of tubing in heat exchangers. AKMEC’s testing devices are portable, contact-less and provide immediate feedback. The eddy current instrument at AKMEC is capable of operating at test frequencies up to 8 kHZ with a high speed of 2-Meter Per Second Inspection Speed.

Pre-requisites: Prior to the testing, all tubes must be cleaned. Cleaning shall be done using hydro jetting with typical cleaning pressures between 280kg/m² and 560 kg/cm². The adequacy of cleaning must be checked by inserting an appropriate dummy probe / rod.

Equipment: The electronic apparatus used will be capable of energizing the test coil with alternating currents of suitable frequencies (for example 1 to 125 kHz) and will be capable of sensing the changes in the electromagnetic response of the coils. The instrument will be balanced with the probe in a defect free position of the calibration tube. The spot will be central on the impedance plane display of screen for differential testing and at one division from the top of the screen for absolute testing.

Evaluation: The expert Eddy Current technicians of AKMEC take the flaw signals generated in tubes under test and then compare it with the signals of the calibration standard.

3. 3 Remote Field Eddy Current Testing (RFET)

Remote field testing (RFT) is an electromagnetic method of non-destructive testing used by AKMEC for the detection and sizing of wall thinning caused by corrosion, erosion, wear, pitting and baffle cuts in ferromagnetic tubes of boilers, feed water heaters, air coolers, and carbon steel heat exchangers.

​

Remote field eddy current testing (RFET) and near field testing (NFT) are used in combination or individually depending upon the capability of testing machine.

3.4 Magnetic Flux Leakage (MFL) Testing

Magnetic flux leakage (MFL) is an electromagnetic non-destructive technique that is used by AKMEC to detect corrosion and pitting.

​

MFL uses a powerful magnet to magnetize the conductive material under test (usually steel). Where there are defects — corrosion or material loss — the magnetic field “leaks” from the steel.

​

- Inspection technique is suitable for wall-loss detection and measurement of sharp defects, such as pitting, grooving, and circumferential cracks.

- Applicable to ferromagnetic tubing.

- Effective for testing aluminum-finned carbon steel tubes because the magnetic field is mostly unaffected by the fins.

- A good Back-up inspection to Remote Field Testing

  - Internal Rotary Inspection System (IRIS)

IRIS is an ultrasonic method for the nondestructive inspection of tubes. The IRIS probe is inserted into a tube that is flooded with water, and the probe is pulled out slowly as the data is displayed and recorded. The ultrasonic beam allows detection of metal loss from the inside and outside of the tube wall.

ADVANTAGES

- IRIS can provide the location of flaw along the length as well as the circumference of the tube

- Defect detection capability for IRIS is better than RFET (in ferrous materials) but it is inferior to ECT (in non-magnetic materials) as IRIS is not as capable of detecting smaller defects when compared to ECT

LIMITATIONS:

- The major disadvantage is that it is a very slow technique (scanning speed @ 1 inch per second), but accurate in wall measurement of ferrous tubes in range of 0.15mm

- IRIS inspection requires a better cleaning of surface than eddy current testing

  - Boroscopic / Videoscopic Inspection

AKMEC offers Boroscopic / videoscopic inspection is a nondestructive testing technique that uses a small camera to visually inspect hard reach zones to find surface defects, general condition, degradation, blockages & foreign materials in Electrical generators & transformers , Boilers, pressure vessels, pipelines ,Gas , stem & wind turbines, condenser tubes , drums , pumps, heat exchangers & various plant components without teardown. They can be used to check pitting, cracks, corrosions, erosions, weld defects & other problems quickly & easily.

3.7 Thermography

AKMEC uses infrared thermography tests to find temperature anomalies present in the equipment during their operation. This is based on remote viewing and is a non-contact method of testing. As a recent addition to the NDE. Any hot object that emits heat radiation is captured with an infrared sensor, which picks up the radiation to form the image of the hot body. The hot and cold regions on the surface can be analyzed for the healthy condition of the object. Thermography is useful in applications such as deposits or blockages in pipelines carrying hot or cold fluids, refractory or insulation deterioration in furnaces, boilers, heaters, converters, piping and pipeline etc.

ADVANTAGES

- The reformers, furnaces and boilers are grateful objects to inspect chocking, deposits and hot/cold spot

- Thermography helps in easily determining hot-spots (overheating) especially on electrical systems which can produce catastrophic hazards

- Qualified team of inspectors to carry out thermography service​

LIMITATIONS

- Required trained and skilled technician for thermography.

- Effective parameters are required to set the limit like; temperature range, emissivity and focal distance etc.

AKMEC offers a wide range of services to fit your needs. We will cater to our client’s specific requirements so please contact us today on info@akmecgroup.com to discuss how AKMEC can assist your company’s needs.